| Mini Project Kinematic calibration |  |
| --- | --- |
| Team members Last name; First name; Last name; First name. | Students, third if any |
| Prepared by: Name; Signature; Date. | Supervisor |
| Controlled by: Name; Signature; Date. | First team member |
| Approved by: Name; Signature; Date. | Second team member, third if any |
| Tests results confirmed by: Name; Signature; Date. | First team member |
| Tests results confirmed by: Name; Signature; Date. | Second team member third if any |
| Tests results approved by: Name; Signature; Date. | Supervisor |
|  | Sion, April 20, 2026 |
| Performance Qualification |  |

# Camera calibration.

| PQ ID | Reset the system |
| --- | --- |
| 1 | From aborted state in execute mode after resetting the PLC program. One team member performs a cold reset and starts in a browser connected to CtrlX OS / /plc/status/applications: The state is aborted in execute mode yes / no? … |

# Camera calibration.

| PQ ID | Validate calibration |
| --- | --- |
| 2.1 | The test engineer presses the sequence of hardware buttons: Reset – Reset - Start The position of the camera is on the QR code, (y/n)? … |
| 2.2 | An indicator prompts you to validate the calibration (y/n)?... |
| 2.3 | The test engineer stops the system and presses the sequence Reset – Reset – Start, there is no more prompts to validate the calibration (y/n)?... |

# Pick and place sequence

| PQ ID | PackML command Stop |
| --- | --- |
| 3.1 | From aborted state, if calibration has been validated, the operators inserts a tube on nest position: (nest position)?... The operation presses the sequence Reset – Reset – Start, insert the nest position above in the User Interface and presses the Start Pick button. The robot picks the tube from position (position)?... The robot is in high position with the tube in the gripper (y/n)? … |
| 3.2 | There is a tube in the gripper, the operator selects the nest number (nest position)?.... The operator presses the Start Place button. The robot places the tube in the nest number (number)?.... The z axis is in up position (y/n)? There is a tube in the gripper (y/n)? |

# Fast commissioning

| PQ ID | Check Hold |
| --- | --- |
| 4,1 | The supervisor opens the door, move the plate from n centimetres to the centre of the cell: (Number of centimetres)?.... |
| 4.2 | The supervisor starts a chronometer. One team member modifies the PCS, enable or disable it with some buttons. Done (y/n)?... One team member reset the PLC using the sequence descried in PQ ID 1, Done (y/n)?... One team member performs a camera calibration validation as described in PQ ID, Done (y/n)?... Write the time of the chronometer here in minutes, seconds:…. |
| 4.3 | The supervisor places a tube in the nest number (nest number) ?.... The supervisor starts a pick sequence from the nest number (nest number) ?.... The robot picks the tubs (y/n)?... |